Wind Energy Conversion Systems

Assignment 1

Muhammad Shamaas

ID # 2018-MS-EE-4

Introduction

This case study investigates the dynamic performance of a 2.3 MW, 690 V, 50 Hz, 1512 rpm squirrel-cage induction generator wind energy system during system start-up, and verifies that a large SCIG cannot be directly connected to the grid due to the excessive inrush current and torque oscillations. The shaft of the generator is coupled to the wind turbine through a gearbox. During the system start-up, the turbine and generator are brought by the wind to a certain speed, at which the generator is connected to the grid of 690 V/50 Hz by a switch. The investigation is carried out in two cases: Dynamic Performance of SCIG with Direct Grid Connection and Direct Grid Connection of SCIG with constant Rotor Speed.

The dq-axis model of the induction generator can be obtained by decomposing the voltage, current and flux linkage space-vectors into their corresponding d- and q-axis components.

xdxq=23cosθcos⁡(θ-2π3)cos⁡(θ-4π3)-sinθ-sin⁡(θ-2π3)-sin⁡(θ-4π3)xaxbxc

The αβ stationary reference frame is derived from dq-axis rotating reference frame by putting θ = 0 and dθdt=ω=0. The transformation of three-phase variables in the stationary reference frame into the two-phase variables also in the stationary frame is often referred to as abc/αβ transformation:

xαxβ=231-12-12032-32xaxbxc

Similarly, the two-phase to three-phase transformation in the stationary reference frame, known as αβ/abc transformation, can be performed by:

xaxbxc=10-1232-12-32xαxβ

Part 1: Dynamic Performance of SCIG with Direct Grid Connection

The wind turbine is initially in a parking mode with the blades pitched out of the wind. When the wind speed reaches an operative level, the blades are pitched into the wind slightly, and wind turbine and generator start to rotate slowly. When the generator is accelerated close to the rated speed of 1450 rpm (0.959 pu), the circuit breaker is closed and the generator is directly connected to the grid.

SCIG dq-axis Reference Frame Model Equations

- Flux Linkage Equations:

λds= (vds-Rs ids+ωλqs)/S

λqs= (vqs-Rs iqs-ωλds)/S

λdr= (vdr-Rr idr+(ω-ωr)λqr)/S

λqr= (vqr-Rr iqr-(ω-ωr)λdr)/S

- Current Equations

ids=(Lrλds-Lmλdr)/D1

iqs=(Lrλqs-Lmλqr)/D1

idr=(-Lmλds+Lsλdr)/D1

iqr=(-Lmλqs+Lsλqr)/D1

D1=LrLs-Lm2

- Motion and Torque Equations

ωr=PJS(Te-Tm)

Te=3P2(iqsλds-idsλqs)

SCIG Machine Constants

- Rated Line-Line Voltage

Vrated,LL=690 V(rms)

- Rated Stator Current

Irated=2168 A(rms)

- Number of Pole Pairs

P=2

- Rated Stator Frequency

fe=50 Hz

- Stator Winding Resistance

Rs=1.102mΩ

- Rotor Winding Resistance

Rr=1.497mΩ

- Stator Leakage Inductance

Lls=0.06492mH

- Rotor Leakage Inductance

Llr=0.06492mH

- Magnetizing Inductance

Lm=2.1346mH

- Moment of Inertia

J=1200 kg.m2

- Rated Rotor Speed

nm,rated=1512rpm

ωm,rated=2π60nm,rated=158.336269741 rad/s

ωr,rated=Pωm,rated=316.672539482 rad/s

- Synchronous Speed

nsynchronous=60Pfe=1500rpm (0.992 pu)

Ls=Lls+Lm=0.06492mH+2.1346mH=2.19952mH

Lr=Llr+Lm=0.06492mH+2.1346mH=2.19952mH

D1=LrLs-Lm2=28.13710704000001*10-6

Initial Conditions (t = 0-)

In this case, the IG model in the stationary reference frame should be used, which was realized by setting the speed of the arbitrary reference frame to zero (w = 0). The dq-axis rotor voltages are set to zero for simulation of squirrel-cage induction generators.

- The reference frame is stationary

ω=0

- Electrical Frequency

fe=50 Hz

- Rotor Speed

nm=1450rpm=0.959 pu

ωm=2π60nm=151.843644924 rad/s=0.959 pu

ωr=Pωm=303.687289847 rad/s=0.959 pu

- Stator Terminals are open

ids=iqs=0

- Rotor voltages are zero

vdr=vqr=0

- Electromagnetic Torque is zero

Te=3PLm2iqsidr-idsiqr=0

The Rotor is slowly accelerated to a constant speed of ωm(t=0-)=0.959 pu by the wind Mechanical Torque Tm. It is assumed that at t = 0-:

- The rotor is not accelerating

Tm=0

dωrdt=0

- Speed Voltages are constant

d(ωλqs)dt=d(ωλds)dt=d(ω-ωrλdr)dt=d(ω-ωrλqr)dt=0.

- Stator and Rotor currents are constant

didsdt=diqsdt=didrdt=diqrdt=0

- Stator and Rotor flux linkages are constant

dλdsdt=dλqsdt=dλdrdt=dλqrdt=0.

- Stator voltages are zero

vds=Rs ids+dλdsdt-ωλqs=0

vqs=Rs iqs+dλqsdt+ωλds=0

- Rotor and Stator Flux Linkages are zero

λds=λqs=λdr=λqr=0

because Te=0

Supply Voltage (t = 0)

fe=50 Hz

|vLL|=690 V(rms)

vas=|vLL|3<0°=398.371685741 <0° Vrms

vbs=|vLL|3<-120°=398.371685741 <-120° Vrms

vbs=|vLL|3<-240°=398.371685741 <-240° Vrms

Amplitude of Phase Voltages=398.371685741*2= 563.38264084 V

The default AC voltage sources in Simulink are Sinusoidal Sources. In order to convert the voltages into cosines, the phase of each voltage is advanced by 90°.

Phase a voltage: Amplitude: 563.38264084 V, phase angle: 90°

Phase b voltage: Amplitude: 563.38264084 V, phase angle: -30°

Phase c voltage: Amplitude: 563.38264084 V, phase angle: -150°

Simulink Model

The input variables of the model include the dq-axis stator voltages vds and vqs, rotor voltages vdr and vqr, the mechanical torque Tm, and the speed of the arbitrary reference frame w. The output variables are dq-axis stator currents, ids and iqs, the electromagnetic torque Te, and the mechanical speed wm of the generator.

- Block diagram for dynamic simulation of SCIG with direct grid connection.

- SCIG System

- Block diagram for dynamic simulation of an induction generator in the arbitrary reference frame

- Reference Frame Transformations

Assuming a three-phase balanced grid, the grid voltages vas, vbs, and vcs in the stationary frame are transformed to the two-phase voltages vα and vβ in the αβ stationary frame through the abc/αβ transformation. The simulated αβ-axis stator currents ids and iqs are also in the stationary frame, which are transformed to the three-phase currents ias, ibs, and ics by the αβ/abc transformation. The three-phase stator voltages are transformed to the two-phase voltages via the abc/αβ transformation and the calculated two-phase stator currents are transformed back to the three-phase stator currents via the αβ/abc transformation.

abc/αβ Transformation

αβ/abc Transformation

- WECSConstants.m

Results

- Currents (ias, ibs, ics)

- Electromagnetic Torque

- Rotor Mechanical Speed

During the system transients, a high inrush current flows into the generator and a DC offset current appears in each of the stator currents ias, ibs, and ics, but the sum of these offset currents is zero due to a three-phase balanced system. As a rotating magnetic field is being built and generator core is being magnetized by the stator current, an electromagnetic torque Te is produced. Since the generator operates below synchronous speed in motoring mode, it produces a positive torque that accelerates the turbine. The generator finally reaches the synchronous speed of 1500 rpm (0.992 pu) at t = 0.84 sec, at which it enters the steady-state operation with Te= Tm = 0. The direct connection of the generator to the grid during the system start-up causes excessive inrush currents with peak values of 8.3 per unit (pu), high electromagnetic torque (2.7 pu, peak), as well as high torque oscillations. It can be concluded that the direct grid connection of the SCIG during the system startup cannot be used in practice, especially for the large megawatt turbines.

Part 2: Direct Grid Connection of SCIG with constant rotor speed

The second part of the case study investigates the transients during the direct grid connection of the generator when the speed of the generator is brought by the wind exactly to the synchronous speed of 1500 rpm (0.992 pu). It is assumed that the combined moment of inertia of the generator, gearbox, and blades are very large, such that the rotor speed is kept constant during the electric transients. In this case, the motion equation is simplified to d(wr)/dt = 0 and wr = ωs. The rotor speed wr is then used as one of the system input variables. As a result, the IG simulation block diagram should be slightly modified to accommodate the changes.

Initial Conditions (t = 0-)

In this case, the IG model in the stationary reference frame should be used, which was realized by setting the speed of the arbitrary reference frame to zero (w = 0). The dq-axis rotor voltages are set to zero for simulation of squirrel-cage induction generators.

- The reference frame is stationary

ω=0

- Electrical Frequency

fe=50 Hz

- Rotor Speed

nm=1500rpm=0.992 pu

ωm=2π60nm=157.079632679 rad/s=0.992 pu

ωr=Pωm=314.159265359 rad/s=0.992 pu

- Stator Terminals are open

ids=iqs=0

- Rotor voltages are zero

vdr=vqr=0

- Electromagnetic Torque is zero

Te=3PLm2iqsidr-idsiqr=0

The Rotor is slowly accelerated to a constant speed of ωm(t=0-)=0.992 pu by the wind Mechanical Torque Tm. It is assumed that at t = 0-:

- The rotor is not accelerating

Tm=0

dωrdt=0

- Speed Voltages are constant

d(ωλqs)dt=d(ωλds)dt=d(ω-ωrλdr)dt=d(ω-ωrλqr)dt=0.

- Stator and Rotor currents are constant

didsdt=diqsdt=didrdt=diqrdt=0

- Stator and Rotor flux linkages are constant

dλdsdt=dλqsdt=dλdrdt=dλqrdt=0.

- Stator voltages are zero

vds=Rs ids+dλdsdt-ωλqs=0

vqs=Rs iqs+dλqsdt+ωλds=0

- Rotor and Stator Flux Linkages are zero

λds=λqs=λdr=λqr=0

because Te=0

Supply Voltage (t = 0)

fe=50 Hz

|vLL|=690 V(rms)

vas=|vLL|3<0°=398.371685741 <0° Vrms

vbs=|vLL|3<-120°=398.371685741 <-120° Vrms

vbs=|vLL|3<-240°=398.371685741 <-240° Vrms

Amplitude of Phase Voltages=398.371685741*2= 563.38264084 V

The default AC voltage sources in Simulink are Sinusoidal Sources. In order to convert the voltages into cosines, the phase of each voltage is advanced by 90°.

Phase a voltage: Amplitude: 563.38264084 V, phase angle: 90°

Phase b voltage: Amplitude: 563.38264084 V, phase angle: -30°

Phase c voltage: Amplitude: 563.38264084 V, phase angle: -150°

Simulink Model

The input variables of the model include the dq-axis stator voltages vds and vqs, rotor voltages vdr and vqr, the mechanical torque Tm, and the speed of the arbitrary reference frame w. The output variables are dq-axis stator currents, ids and iqs, the electromagnetic torque Te, and the mechanical speed wm of the generator.

- Block diagram for dynamic simulation of SCIG with direct grid connection.

- SCIG System

- Block diagram for dynamic simulation of an induction generator in the arbitrary reference frame

- WECSConstants.m

Results

- Currents

- Electromagnetic Torque

A high inrush current is drawn by the generator, with a peak value is 8.3 pu. The high amplitude of the stator currents causes oscillations in the generator torque Te with a peak of 1.63 pu. Compared with the previous case, the transient process is faster due to the constant rotor speed that eliminates the motion equation in the simulation. It can be concluded that the direct connection of a SCIG to the grid is not allowed in practice due to the excessive stator current and torque oscillations